Instruction to update Radtel RT-950 or RT-950 PRO

- Click the update tool (rt-950 and  rt-950 pro use same upgrade tool)

- Load the firmware file RT_950.BTF and select COM port

- Turn ON the power (don’t need to press any combine keys)

4.The screen will show a update logo and press Upgrade Mode to upgrade firmware

After finish, it shows 100% and radio power on again

The update have been completed!

How to fix it if the radio become brick after upgrade ??? Don’t worry, please follow this steps:

1.Remove the battery and wait for 10 seconds

- Turn off the power

- holding down side keys 3 and 4, while turn on the power enter the Update interface

- click the [Flashing Mode] button, and wait for 100% completion

After finish, it shows 100% and radio power on again